# EXPERIMENT NO. 2

AIM: Practically implement and test the cross-wired cable and straight through cable         using clamping tool and network lab cable tester.

MATERIAL AND REQUIREMENTS:

- Ethernet cables (bulk cable or pre-terminated cables)

- RJ-45 connectors

- Clamping tool (crimper)

- Cable tester (with two RJ-45 connectors)

- Wire stripper and cutter

- Marker and labels

- Safety goggles (for eye protection)

FIG: RJ-45 CONNECTOR

INTRODUCTION:

In computer networking, it's essential to understand the distinction between cross-wired (crossover) and straight-through cables. Cross-wired cables are used to connect similar devices (e.g., two computers), while straight-through cables are used to connect different devices (e.g., a computer to a switch). This practical will guide students through the process of creating and testing both cable types.

PROCEDURE:

Part 1: Cross-Wired (Crossover) Cable

Cable Preparation:

- Start by cutting a length of Ethernet cable. Strip about 1 inch (2.5 cm) of the cable's outer insulation to expose the inner twisted pairs.

Wire Arrangement:

- Follow the T568A or T568B wiring standard for Ethernet cables.

- Cross-wire the transmit and receive wires at one end of the cable, as per the crossover cable standard (e.g., T568A at one end and T568B at the other).

Connector Termination:

- Insert the wires into an RJ-45 connector, ensuring proper alignment.

- Use a clamping tool to crimp the connector securely.

- Repeat the process for the other end, ensuring the same cross-wiring scheme.

Part 2: Straight-Through Cable

Cable Preparation:

- Cut another length of Ethernet cable. Strip about 1 inch (2.5 cm) of the cable's outer insulation to expose the inner twisted pairs.

Wire Arrangement:

- Follow either the T568A or T568B wiring standard for both ends of the cable.

Connector Termination:

- Insert the wires into an RJ-45 connector at one end, following the chosen standard.

- Use a clamping tool to crimp the connector securely.

- Repeat the process for the other end, ensuring the same wiring standard is used.

Part 3: Testing Cables

Lab Cable Tester:

- Plug one end of the created cross-wired cable into one RJ-45 connector of the lab cable tester.

- Plug the other end into the second RJ-45 connector of the tester.

- Check the tester's display for a successful connection.

- Repeat Testing for Straight-Through Cable:

- Test the straight-through cable in the same manner using the lab cable tester.

- Verify that it also shows a successful connection.